$Id: memdisk.doc,v 1.1 2001/12/11 21:17:42 hpa Exp $

[This documentation is rather crufty at the moment.]

MEMDISK is meant to allow booting legacy operating systems via PXE,

and as a workaround for BIOSes where ISOLINUX image support doesn't

work.

MEMDISK simulates a disk by claiming a chunk of high memory for the

disk and a (very small - 2K typical) chunk of low (DOS) memory for the

driver itself, then hooking the INT 13h (disk driver) and INT 15h

(memory query) BIOS interrupts.

To use it, type on the SYSLINUX command line:

memdisk initrd=diskimg.img

... where diskimg.img is the disk image you want to boot from.

[Obviously, the memdisk binary as well as your disk image file need to

be present in the boot image directory.]

... or add to your syslinux.cfg/pxelinux.cfg/isolinux.cfg something like:

label dos

kernel memdisk

append initrd=dosboot.img

Note the following:

a) The disk image should *not* be compressed.

b) If the disk image is one of the following sizes, it's assumed to be a

floppy image:

368,640 bytes -  360K floppy

737,280 bytes -  720K floppy

1,222,800 bytes - 1200K floppy

1,474,560 bytes - 1440K floppy

2,949,120 bytes - 2880K floppy

For any other size, the image is assumed to be a hard disk image (and

should typically have an MBR and a partition table.)  Currently, the hard

disk is always given the geometry of 16 heads, 63 sectors (the number of

cylinders is derived from the image size) and should therefore be an even

multiple of 516,096 bytes; in a future version I plan to examine the

partition table to deduce the CHS geometry.

SYSLINUX 1.65-pre4 does not contain what I would call a "release

quality" version of MEMDISK, but it does seem to work (I have

successfully booted DOS over the net using PXELINUX with it.)  It

contains an awful lot of debugging messages at this point, for one

thing.

If you want to recompile it from sources, you will need to make sure that

you have a fairly recent binutils installed.  I have been using the

binutils-2.11.90.0.8-9 RPM from RedHat.